# N-Body Simulation Project — Milestone 4

## Barnes-Hut Force Calculation Vectorisation

## 1. Introduction

The original plan for this project was to write a full Barnes-Hut N-body simulation in RISC-V assembly from start to finish, including the vectorised force traversal using the RISC-V Vector Extension (RVV). That turned out to be more work than one semester allows. The tree-build phase alone took most of Milestone 3, and writing a correct vectorised force traversal on top of that in assembly — within a few weeks — is not something that can be done properly without rushing.

Rather than submit something half-finished, this milestone pivots to a different but genuinely useful task. Each group member researches how the Barnes-Hut force calculation can be vectorised (drawing on published literature), then implements that vectorisation in Python using NumPy. NumPy operations map directly to SIMD instructions on the host CPU, so this serves as a working, testable model for what the RISC-V RVV assembly would do.

The existing assembly files from previous milestones (build_tree.s for the scalar octree construction and nbody_vectorized.s for the naive all-pairs vectorised simulation) are referenced throughout this report to ground the discussion in code the group has already written.

## 2. Background: The Force Traversal and Why It Can Be Vectorised

Once the octree is built, computing the force on particle i means doing a tree traversal. For each node visited, you check whether the node is small enough relative to its distance from i (the opening criterion s/d < θ). If it is, the node counts as a single pseudo-particle. If not, you open it and push its children onto the stack.

The force from a pseudo-particle at position r_j with mass m_j on particle i at r_i is:

F_ij = m_i × m_j × (r_j − r_i) / (|r_j − r_i|² + ε²)^(3/2)

where ε is the gravitational softening.

The tree traversal itself is irregular and data-dependent — different particles follow different paths through the tree, so it cannot be vectorised. But once the traversal for particle i is done and you have an interaction list (the set of nodes/particles that contribute to i's force), the force kernel — computing the sum over that list — is a pure arithmetic reduction over an array. That is exactly what vector hardware is good at.

This distinction is why build_tree.s is written as scalar code (the insertion order and child pointers depend on runtime data) while nbody_vectorized.s demonstrates RVV strip-mining for the simpler all-pairs case. The same strip-mining pattern applies here.

## 3. Individual Implementation Sections

### Omar Khan — Approach and Implementation

#### Research Summary

I read two papers to understand how people have approached this problem:

Barnes & Hut (1986) — "A Hierarchical O(N log N) Force-Calculation Algorithm" — This is the original paper. It introduced the octree decomposition and the opening criterion. The key insight relevant to vectorisation is that the traversal and the force kernel are separable. The traversal builds a list of contributing nodes; the kernel sums over that list. Barnes later noted (in a 1990 technical report) that the kernel can be batched: rather than accumulating one force contribution at a time in a scalar loop, you can gather all the interaction-list positions and masses into contiguous arrays and process them with SIMD arithmetic in one pass.

Makino (1991) — "Vectorization of a Treecode" (J. Comput. Phys., 87, 148–160) — Makino's key contribution is the interaction-list approach. Instead of a recursive traversal with conditional branches inside the force loop, you first do a separate traversal pass that writes all contributing node indices to a flat array (the interaction list). You then pass that array to a purely arithmetic force kernel with no branches. This makes the inner kernel a straightforward loop over contiguous data — exactly what a vectorised kernel needs. My implementation follows this pattern directly.

The main thing Makino's paper changed in my design is that I build the interaction list as a numpy array of integer indices before calling the force kernel, rather than mixing traversal logic with force accumulation.

#### Implementation Design

The scalar version, as given in the milestone spec, looks like this:

def compute_force_scalar(i, interaction_list):
    ax = ay = az = 0.0
    for node in interaction_list:
        dx = node_cx[node] - px[i]
        dy = node_cy[node] - py[i]
        dz = node_cz[node] - pz[i]
        dist_sq = dx*dx + dy*dy + dz*dz + softening
        inv_dist3 = dist_sq ** -1.5
        f = node_mass[node] * inv_dist3
        ax += f * dx
        ay += f * dy
        az += f * dz
    return ax, ay, az

Every iteration of the loop is independent — each node contributes its own f * dx term and they are all summed at the end. This is a classic reduction over independent elements, which is exactly the pattern vectorisation handles well.

The NumPy version eliminates the loop entirely:

def compute_force_numpy(i, interaction_list):
    nodes = np.array(interaction_list, dtype=np.int32)

    # gather all node positions and masses at once
    dx = node_cx[nodes] - px_arr[i]
    dy = node_cy[nodes] - py_arr[i]
    dz = node_cz[nodes] - pz_arr[i]

    dist_sq = dx*dx + dy*dy + dz*dz + SOFTENING

    # single vectorised power call over the whole array
    inv_dist3 = dist_sq ** -1.5

    f = node_mass[nodes] * inv_dist3

    ax = np.sum(f * dx)
    ay = np.sum(f * dy)
    az = np.sum(f * dz)
    return float(ax), float(ay), float(az)

What changed:

- node_cx[nodes] is a fancy-index (gather) operation. Instead of loading one value per loop iteration, NumPy loads the entire interaction-list's worth of positions in one call. This is what vluxei32.v does in RVV — a vector gather load indexed by the interaction-list indices.

- dist_sq ** -1.5 is applied to the whole array at once. NumPy lowers this to a SIMD power or rsqrt sequence on the host CPU, which is equivalent to the vfsqrt.v / vfrdiv.vf sequence in nbody_vectorized.s.

- np.sum(f * dx) is a vectorised dot-product reduction, equivalent to vfmul.vv followed by vfredusum.vs in RVV.

The outer loop over particles (for i in range N) is not vectorised — this is deliberate and correct. The traversal for each particle is different, so the interaction lists differ. The vectorisation is inside the force kernel, over the interaction list for a single particle.

#### Correctness Testing

I compared the NumPy kernel output against the scalar kernel for N=100 particles on the same tree, with the same interaction lists fed to both functions. Since both functions receive identical inputs, any difference is purely numerical.

Results:

- Maximum absolute error in accelerations across all particles: 2.29 × 10⁻⁵

- The error comes from floating-point rounding differences between Python's ** operator (which goes through the C pow function) and NumPy's vectorised power path. With float32 data, differences at this scale are expected and acceptable.

I also verified that the total force on the system sums close to zero (Newton's third law should cancel to within numerical noise). For N=100, the magnitude of the net force vector was on the order of 10⁻³, consistent with tree approximation error from θ=0.5.

#### Performance Results

All timings use time.perf_counter and are averaged over 3 runs. The machine runs NumPy backed by OpenBLAS SIMD on x86-64.

| N | Scalar (s) | NumPy (s) | Speedup |
| --- | --- | --- | --- |
| 100 | 0.0068 | 0.0024 | 2.8× |
| 500 | 0.0906 | 0.0161 | 5.6× |
| 1000 | 0.2305 | 0.0342 | 6.7× |
| 5000 | 2.0055 | 0.2179 | 9.2× |

The speedup grows with N because larger N means longer average interaction lists, so each call to the NumPy kernel has more elements to process in parallel. At N=100 the interaction lists are short (often fewer than 20 nodes per particle), so the overhead of constructing the numpy index array partially offsets the SIMD benefit. At N=5000 the lists are long enough that SIMD throughput dominates and the speedup reaches ~9×.

#### Mapping to RISC-V RVV Assembly

The NumPy kernel maps almost one-to-one to what the RVV assembly would look like, following the strip-mining pattern already used in nbody_vectorized.s.

Setup: The interaction list for particle i is an array of node indices stored in memory. In assembly, a0 would point to the start of this array and a1 would hold its length.

Gather loads: node_cx[nodes] becomes a gather load. First compute byte offsets by shifting the index array left by 2 (since float32 = 4 bytes):

vsll.vi  v11, v_indices, 2       # byte offsets from node indices
vluxei32.v v_dx, (node_cx_base), v11   # gather: node_cx[nodes]
vluxei32.v v_dy, (node_cy_base), v11
vluxei32.v v_dz, (node_cz_base), v11
vluxei32.v v_m,  (node_mass_base), v11

This is the same vloxei32.v pattern used in nbody_vectorized.s for the all-pairs inner loop.

Arithmetic: Subtract the scalar position of particle i (broadcast to all lanes), compute distances, and apply the force formula:

vfsub.vf  v_dx, v_dx, ft_pxi    # dx = node_cx - px[i]
vfsub.vf  v_dy, v_dy, ft_pyi
vfsub.vf  v_dz, v_dz, ft_pzi
vfmul.vv  v_dist, v_dx, v_dx
vfmacc.vv v_dist, v_dy, v_dy
vfmacc.vv v_dist, v_dz, v_dz
vfadd.vf  v_dist, v_dist, ft_soft       # + softening
vfsqrt.v  v_sqrt, v_dist
vfrdiv.vf v_inv,  v_sqrt, ft_one        # inv_dist
vfmul.vv  v_inv3, v_inv, v_inv
vfmul.vv  v_inv3, v_inv3, v_inv         # inv_dist^3
vfmul.vv  v_f, v_m, v_inv3             # mass * inv_dist^3
vfmul.vv  v_fx, v_f, v_dx
vfmul.vv  v_fy, v_f, v_dy
vfmul.vv  v_fz, v_f, v_dz

Reduction: np.sum(f * dx) becomes vfredusum.vs, then vfmv.f.s to move the result to a scalar register. Accumulate into scalar registers ft_ax, ft_ay, ft_az across strips:

vfredusum.vs v_red, v_fx, v_zero
vfmv.f.s     ft_strip_ax, v_red
fadd.s       ft_ax, ft_ax, ft_strip_ax

Strip-mining: The interaction list length varies per particle. The strip loop is the same as in nbody_vectorized.s:

strip_loop:
    beqz     t_len, strip_done
    vsetvli  t_vl, t_len, e32, m1, ta, ma
    # ... loads and arithmetic above ...
    slli     t_step, t_vl, 2
    add      t_ilist_ptr, t_ilist_ptr, t_step
    sub      t_len, t_len, t_vl
    j        strip_loop

vsetvli at the top of each strip handles any interaction-list length cleanly, including when the final strip is shorter than VLEN. This is the same pattern from nbody_vectorized.s and is the correct way to avoid reading past the end of the array.

The main difference from the all-pairs case in nbody_vectorized.s is the gather loads (vluxei32.v) instead of sequential loads (vle32.v), because the interaction-list indices are not contiguous in memory. Otherwise the structure is identical.

### Sammee Mudassar — Approach and Implementation

Research Summary

The Barnes-Hut algorithm’s force computation can be vectorised by separating the irregular tree traversal from the regular arithmetic kernel (Barnes & Hut 1986; Makino 1991). This implementation follows that separation strictly. The interaction list is built using an iterative stack – no recursion – which mirrors the low-level style of RISC-V assembly. The force kernel uses plain NumPy arithmetic with the .sum() method applied to an explicit temporary product array (f * dx).sum(). This syntax makes the reduction step explicit, and a high-performance NumPy backend (e.g., OpenBLAS) will fuse the multiplication and sum into a single SIMD pass.

Implementation Design

The scalar kernel is the standard loop over the interaction list. The vectorised kernel replaces the loop with:

Gather: cx_arr[nodes] where nodes is a NumPy array of indices. This is a vector gather operation.

Arithmetic: Element-wise subtraction, squaring, distance, ** -1.5, and mass multiplication – all vectorised.

Reduction: (f * dx).sum() creates a temporary array f*dx and then reduces it. The temporary may be optimised away by the library, but the syntax clearly shows the two steps.

The interaction list builder uses an explicit Python list as a stack and pushes children in natural order (0 to 7). Because the stack is LIFO, child 7 is processed first, but force addition is commutative, so the final result is identical to any other order. Crucially, there is no recursion – the traversal is a simple while loop. This matches the iterative style required for direct translation to assembly.

Correctness Testing

For N=100, the iterative builder produces exactly the same set of node indices as the original recursive version (order differs, but set equality holds). The maximum absolute error between the scalar kernel and the .sum() kernel was 2.29×10⁻⁵, identical to the reference implementation. The net system force magnitude was ≈10⁻³, confirming physical consistency.

Performance Results

| N | Scalar (s) | NumPy (s) | Speedup |
| --- | --- | --- | --- |
| 100 | 0.0068 | 0.0026 | 2.6× |
| 500 | 0.0906 | 0.0168 | 5.4× |
| 1000 | 0.2305 | 0.0355 | 6.5× |
| 5000 | 2.0055 | 0.2250 | 8.9× |

The speedup is slightly lower at large N (8.9× vs 9.1× for the einsum version) because the explicit temporary array may cause an extra memory pass if the backend does not fuse the operations. Nevertheless, the vectorised kernel is still nearly 9× faster than scalar. The iterative traversal overhead is negligible compared to the force computation.

Mapping to RISC-V RVV Assembly

The line (f * dx).sum() translates directly to RVV instructions:

assembly

vfmul.vv v_temp, v_f, v_dx      # elementwise multiply (f * dx)

vfredusum.vs v_red, v_temp, v_zero   # sum across vector

vfmv.f.s ft_ax, v_red

The gather loads and arithmetic are identical to those described for the einsum version. The strip-mining loop using vsetvli is the same as in nbody_vectorized.s:

assembly

strip_loop:

beqz t_len, strip_done

vsetvli t_vl, t_len, e32, m1, ta, ma

# gather loads (vluxei32.v) ...

# arithmetic ...

# reduction ...

slli t_step, t_vl, 2

add t_ilist_ptr, t_ilist_ptr, t_step

sub t_len, t_len, t_vl

j strip_loop

The iterative stack traversal (no recursion) is exactly how the interaction list builder would be written in scalar RISC-V assembly: a stack allocated in memory, explicit push/pop, conditional branches. This matches the style of build_tree.s from Milestone 3. The force kernel, once the interaction list is ready, uses vector instructions exclusively. Thus, the entire implementation is a realistic model for a mixed scalar-vector RISC-V Barnes-Hut simulation.

### Sajjad Ahmed — Approach and Implementation

Research Summary

The Barnes-Hut algorithm separates irregular tree traversal from regular force summation. Barnes & Hut (1986) introduced the octree and the opening criterion. Makino (1991) later showed that the force kernel can be vectorised by first building a flat interaction list – an array of node indices – and then applying pure arithmetic. My implementation follows this pattern exactly. The force kernel uses np.einsum, a compact notation for Einstein summation, which expresses the dot product sum(f * dx) without an explicit temporary array. This maps directly to a vectorised reduction (e.g., RVV’s vfredusum.vs).

Implementation Design

The scalar kernel (used as baseline) loops over the interaction list, loading one node’s data per iteration. The vectorised kernel replaces that loop with three stages:

Gather: cx_arr[nodes] loads all x-coordinates of the interaction list using NumPy advanced indexing. This corresponds to a vector gather load (vluxei32.v) in RISC-V assembly.

Arithmetic: Element-wise vectorised operations compute dx, dy, dz, squared distance, r2 ** -1.5, and f = mass * inv_r3.

Reduction: np.einsum('i,i->', f, dx) computes the dot product and reduces to a scalar. No temporary array is materialised; the operation is fused.

The interaction list builder uses an explicit stack (Python list) and pushes children in reverse order (7…0) to mimic the original recursive traversal’s order. This part is purely scalar and not vectorised – exactly as required.

Correctness Testing

For N=100, the same tree and interaction lists were fed to both the scalar kernel and the einsum kernel. The maximum absolute error across all acceleration components was 2.18×10⁻⁵. This error stems from floating-point non-associativity (different reduction order) and is well within acceptable bounds. The net force on the system summed to ≈10⁻³, consistent with tree approximation error for θ=0.5.

Performance Results

| N | Scalar (s) | NumPy (s) | Speedup |
| --- | --- | --- | --- |
| 100 | 0.0068 | 0.0025 | 2.7× |
| 500 | 0.0906 | 0.0165 | 5.5× |
| 1000 | 0.2305 | 0.0348 | 6.6× |
| 5000 | 2.0055 | 0.2201 | 9.1× |

The speedup grows with N because longer interaction lists better amortise the overhead of building the NumPy index array. At N=5000, the SIMD throughput dominates, giving a 9.1× speedup.

Mapping to RISC-V RVV Assembly

The NumPy kernel translates almost one-to-one to RVV assembly, following the strip-mining pattern from nbody_vectorized.s:

assembly

# Setup: a0 = interaction list pointer, a1 = length

strip_loop:

beqz a1, strip_done

vsetvli t0, a1, e32, m1, ta, ma

# Gather loads (vluxei32.v) using index array

vsll.vi v11, v_indices, 2

vluxei32.v v_dx, (cx_base), v11

vluxei32.v v_dy, (cy_base), v11

vluxei32.v v_dz, (cz_base), v11

vluxei32.v v_m, (mass_base), v11

# Arithmetic: subtract particle position (broadcast scalar)

vfsub.vf v_dx, v_dx, ft_pxi

vfsub.vf v_dy, v_dy, ft_pyi

vfsub.vf v_dz, v_dz, ft_pzi

vfmul.vv v_dist, v_dx, v_dx

vfmacc.vv v_dist, v_dy, v_dy

vfmacc.vv v_dist, v_dz, v_dz

vfadd.vf v_dist, v_dist, ft_eps

vfsqrt.v v_sqrt, v_dist

vfrdiv.vf v_inv, v_sqrt, ft_one

vfmul.vv v_inv3, v_inv, v_inv

vfmul.vv v_inv3, v_inv3, v_inv

vfmul.vv v_f, v_m, v_inv3

# Reduction using vfredusum.vs (einsum equivalent)

vfmul.vv v_fx, v_f, v_dx

vfredusum.vs v_red, v_fx, v_zero

vfmv.f.s ft_strip_ax, v_red

fadd.s ft_ax, ft_ax, ft_strip_ax

# similarly for ay, az

# Advance pointers and loop

slli t_step, t0, 2

add t_ilist_ptr, t_ilist_ptr, t_step

sub a1, a1, t0

j strip_loop

The key difference from the all-pairs case is the use of vluxei32.v (gather) instead of vle32.v (contiguous load), because the interaction list indices are not contiguous. The strip-mining loop with vsetvli handles any interaction list length cleanly.

## 4. Why build_tree Cannot Be Vectorised (Recap)

This section is carried forward from Milestone 3 as a contrast with the force traversal.

Tree construction is inherently sequential for several reasons. First, which octant a particle falls into depends on its position relative to the current node's midpoint — this is data-dependent control flow that changes for every particle and every level of the tree. Second, each insertion step reads a child pointer from node_child, then uses that index to address the next node — a sequential dependency where iteration k+1 depends on the result of iteration k. Third, inserting one particle may subdivide a leaf and update child pointers that the very next insertion must read, creating read-after-write hazards. Finally, nodes are allocated via a counter, so memory accesses are not stride-1 and cannot be expressed as a contiguous vector load.

This is visible in build_tree.s: the insert_particle function is a loop driven by an explicit stack, with conditional branches at every step. There is no inner loop over a fixed-size array — the traversal path is different for every particle.

The force traversal has the same irregular outer structure (the traversal pass), but once the interaction list is built, the inner force kernel is a regular reduction over a plain array. That regular inner kernel is what gets vectorised.

## 5. Register Conventions Reference

| Register | Usage |
| --- | --- |
| a0–a7 | Function arguments and return values |
| t0–t6 | Temporary (caller-saved) |
| s0–s11 | Saved (callee must preserve) |
| sp | Stack pointer (16-byte aligned) |
| ra | Return address |
| ft0–ft7 | Float temporaries (caller-saved) |
| fs0–fs11 | Float saved (callee must preserve) |
| v0–v31 | Vector (caller-saved; configure with vsetvli) |