N-Body Gravitational Simulation:

Assembly Implementation and SIMD Vectorization

Team: mubarak paan shop

Muhammad Omar Khan (31599)  |  Sammee Mudassar (30765)  |  Sajjad Ahmed (30406)

# Abstract

This report is about what we did for milestone 2 of the n-body simulation project. In milestone 1 we had written everything in Python. This time we had to take that same naive algorithm and write it again in RISC-V assembly language which was a very different and more difficult experience. Before writing assembly we first had to change our Python code to remove things like append and replace them with fixed size arrays because assembly does not have dynamic lists. After that we wrote the full leapfrog simulation in assembly. To connect the assembly simulation to the Python visualizer we used a thing called memory mapping where both programs can read and write to the same file. We also did three exercises on RISC-V vector instructions (RVV) which is the SIMD extension for RISC-V.

# I. INTRODUCTION

In milestone 1 we built the n-body simulation in Python and compared the naive O(N2) method with the Barnes-Hut O(N log N) method. Milestone 2 is about translating the naive algorithm into assembly language. This was honestly the hardest part of the whole project because assembly does not have any of the things Python has like loops that count automatically, named variables, or functions that save their own state.

In assembly everything is done manually. You load addresses into registers, you calculate offsets yourself by multiplying the index by 4, and if you forget to save a register before calling a function then that register gets overwritten and you get wrong answers with no error message. This made debugging very painful.

The main goals for milestone 2 were to first refactor the Python so it looks more like assembly, then write the full leapfrog in RISC-V assembly and connect it to the visualizer using mmap, and finally do the RVV vector exercises.

# II. CODE REFACTORING

## A. Why We Had to Refactor

The Python code from milestone 1 used append() to add particles to lists one by one as it read from the CSV. This is normal Python but the problem is that assembly does not have dynamic arrays. In assembly, when you write something like px: .space 400 you are saying exactly how many bytes you want and that is it. You cannot make it bigger at runtime.

So we had to change the Python first so that all arrays are pre-allocated with a fixed size before any data is loaded. This way the structure of the Python code matches what we do in assembly.

## B. What We Changed

The main change was replacing the append based loading with index based loading. Instead of m.append(float(row["mass"])) we now do m[i] = float(row["mass"]) where i is a counter that goes from 0 to N-1. The arrays are all created as [0.0] * N before the loop starts. This is equivalent to .space arr_bytes in the data section. The simulation logic (kick, drift, calculate_acceleration) was not changed at all, only the initialization part.

# III. ASSEMBLY IMPLEMENTATION

## A. Overall Structure

The assembly file has a data section where all the constants and the shared memory filename are defined. The positions velocities and masses are in a separate file called mydata.S which we include. The accelerations are in the bss section because they start as zero. The text section has the _start label and the four functions calc_kick, calc_drift, calc_acc.

The _start label does two system calls. First it opens the shared memory file using the openat syscall (number 56). Then it memory maps the file using mmap2 (syscall number 192). The file descriptor is saved in s0 and the mmap address is saved in s1.

## B. The Main Loop

After setup the program enters an infinite loop. It calls calc_kick then calc_drift then calc_acc then calc_kick again. This is the leapfrog Kick-Drift-Kick pattern from milestone 1. Before calling the functions we save s1 on the stack because calc_acc uses s1 internally as a loop counter which would overwrite our mmap address. After the functions return we restore s1.

After the leapfrog step is done we write the new px, py, pz arrays into the shared memory so Python can read them and display the updated positions.

## C. The Kick Function

calc_kick does the half step velocity update which is v += 0.5 * a * dt. We precompute 0.5 * dt once at the start of the function by loading the float bit patterns for dt (0x46070000) and 0.5 (0x3F000000) and multiplying them. Then in the loop for each body we load vx[i] and ax[i], do fmadd.s to compute ax[i] * half_dt + vx[i] and store it back. Same for vy and vz.

fmadd.s does multiply and add in one instruction which is more accurate than doing them separately. This was one of the things we learned about floating point in assembly.

## D. The Drift Function

calc_drift is the position update: p += v * dt. It is basically the same structure as kick but uses the full dt and updates px/py/pz using vx/vy/vz. Not much to say, it is the simpler of the three functions.

## E. The Acceleration Function

calc_acc is the most complicated function. It is a direct translation of the O(N2) double loop from Python. It first zeros out all the acceleration arrays by doing fsw with 0.0 into each ax[i], ay[i], az[i]. We had a bug here at first because we were using ft0 to store the zero but ft0 was not actually set to zero at that point. We fixed it by using fcvt.s.w ft11, zero before the zeroing loop.

The outer loop goes over each body i and loads its position into saved float registers fs0 fs1 fs2. The inner loop goes over every body j and for each pair computes dx dy dz, then r squared plus softening squared, then square root, then force per mass using G * mass[j] / r_soft_cubed, and accumulates into fs3 fs4 fs5. After the inner loop finishes for body i the accumulated accelerations are stored back into the ax ay az arrays.

One important thing is that the partial sums have to be in saved registers (fs registers) because the inner loop uses a lot of temporary ft registers and any of them can get overwritten. If we had stored the accumulator in ft registers they would have been corrupted.

## F. Register Usage

| Register | What we used it for |
| --- | --- |
| s0 | file descriptor from openat |
| s1 | base address of mmap region |
| s2-s4 | base addresses of vx/vy/vz or px/py/pz |
| s5-s7 | base addresses of ax/ay/az |
| s8 | outer loop counter i |
| s9 | loop limit n = 100 |
| fs0-fs2 | position of current body i |
| fs3-fs5 | accumulated acceleration for body i |
| ft0-ft9 | temporary registers for distance and force calculation |

# IV. MEMORY MAPPING AND SYNCHRONIZATION

## A. How mmap Works

Memory mapping is a way to share data between two programs without using normal read and write system calls. When you mmap a file you get a pointer to a region in your virtual memory and any writes to that region automatically go into the file. Another program that has also mmapped the same file can see those writes directly.

We used this so that the assembly simulation can write positions into the file and the Python visualizer can read them and display them. The assembly opens a file called shared.mem and maps 16384 bytes of it. The Python side opens the same file and creates its own mmap object. Both are pointing at the same physical memory.

## B. The Flag for Synchronization

The problem with mmap is that if Python reads while assembly is still writing it can get half-written data, for example px fully written but py still from the last timestep. This would cause the visualization to show wrong positions.

To fix this we put a flag at the very start of the shared memory (the first 4 bytes). Assembly sets this flag to 1 before it starts writing the arrays and sets it back to 0 when it is done. Python checks this flag in a loop and only reads when the flag is 0. This way they do not overlap. It is a simple solution but it worked fine for our purposes.

# V. RISC-V VECTOR EXTENSION AND SIMD EXERCISES

## A. What is SIMD

SIMD means Single Instruction Multiple Data. Normal instructions do one thing at a time, one add, one load. SIMD instructions can do the same operation on many values at the same time. For example instead of adding 16 pairs of numbers one at a time in 16 instructions, a SIMD instruction can do all 16 at once.

RISC-V has its own vector extension called RVV. One interesting thing about it is that unlike Intel SSE which always uses 128 bit registers, RVV uses a variable length model. You tell the hardware how many elements you want to process using vsetvli and the hardware decides how many it can actually do depending on the physical register width. This means the same code works on chips with different vector sizes.

## B. Exercise 1: Clamp

The goal of this exercise was to cap all values in an array at 255. If a value is bigger than 255 replace it with 255, otherwise keep it.

In scalar code this needs an if statement inside the loop. In vector code we do it without any branch. First vsetvli sets how many elements to process. Then vle32.v loads them. Then vmsgt.vx creates a mask where each bit is 1 if the element was greater than 255. Then vmerge.vxm uses that mask to pick 255 for elements that were too big and keep the original for the rest. Then vse32.v stores the result.

## C. Exercise 2: Counting Negatives

The goal was to count how many numbers in an array are less than zero. The scalar version needs a conditional inside the loop and a counter variable. In the vector version vmslt.vx creates a mask where bits are set for negative elements, and then vcpop.m counts how many bits are set in that mask. This collapses the whole vector comparison into a single integer in one instruction which is a lot faster.

## D. Exercise 3: Dot Product

The dot product multiplies two arrays element by element and sums all the results. This is used a lot in machine learning and also relevant to our n-body force calculations. The vector version uses vmul.vv to do all the multiplications at once and vredsum.vs to do a reduction sum that adds everything into one accumulator register. After the loop vmv.x.s extracts the final scalar answer.

## E. Why SIMD Matters for N-Body

The inner loop of calc_acc is basically doing the same floating point operations on many different bodies. This is exactly the kind of problem SIMD is good at. If we had vectorized the inner loop using RVV fmadd.vv we could potentially do 4 to 8 bodies per cycle instead of one. For N=100 that might not make a huge difference but for larger simulations it would matter a lot.

# VI. PROFILING WITH QEMU

QEMU has a plugin system that can count how many instructions the program executes. To profile we made the simulation run only one iteration and then exit instead of looping forever. This gives a clean count for exactly one leapfrog step.

For N=100 the results confirmed what we expected. Almost all the floating point instructions were in calc_acc because it runs N squared = 10000 pairs each with about 20 floating point operations. The kick and drift functions only run N = 100 iterations each so their contribution is basically nothing compared to calc_acc. This means if we ever want to make this faster we should focus all effort on the inner loop of calc_acc.

# VII. IMPLEMENTATION CHALLENGES

## A. Register Clobbering

The biggest problem we kept running into was registers getting overwritten. In Python when you write a variable it stays there until you change it. In assembly all the functions share the same registers so if a function changes s1 and you needed that value it is just gone. We had this problem with s1 which held the mmap address but was also being used as a loop variable inside calc_acc.

The solution is to save registers on the stack before calling functions and restore them after. The pattern is addi sp sp -16, then sw the register, call the function, then lw it back, then addi sp sp 16. We had to do this carefully for every function that could overwrite something important.

## B. Float Constants in Hex

In Python you can just write G = 6.67430e-11 and it works. In assembly you need the actual 32 bit IEEE 754 hex value. We computed these by hand and put them as equ constants at the top of the file. For example dt = 8640.0 becomes 0x46070000. If you get this wrong by even one bit in the exponent field the value becomes completely wrong and the simulation explodes or freezes.

## C. Offset Calculation

Every single array access in assembly requires manually computing the byte offset. Since each float is 4 bytes the offset for element i is i*4. In assembly we use slli t0 s8 2 which shifts left by 2 (same as multiplying by 4) to get the byte offset. If you forget to multiply or use the wrong shift amount you read from the wrong address and get garbage values. This type of bug is very hard to find because there is no error message, the program just gives wrong numbers.

## D. Debugging in General

Assembly debugging is very painful. There are no print statements, no exceptions, no stack traces. When something goes wrong the simulation either crashes or produces wrong positions and you have to go back and check every register and every address manually. We used the LLM as suggested by the TA to check our logic and that helped a lot for catching small mistakes like missing stack saves.

# VIII. RESULTS AND DISCUSSION

The assembly implementation gives the same results as the Python from milestone 1 when tested on the same initial conditions file. The particle trajectories look visually identical in the visualizer which means the force calculations are correct. This was a good sign because it is easy to make small mistakes in assembly that change the physics.

The mmap interface worked correctly. The Python visualizer reads from the shared file smoothly and the flag synchronization stops any problems with Python reading while assembly is still writing. The simulation runs continuously without any crashes.

For the SIMD exercises all three give correct answers. The clamp exercise correctly caps values at 255. The negative count gives the right number for arrays where we know the answer. The dot product gives the correct sum which we verified by computing it manually for small arrays.

One thing we noticed is that the assembly simulation runs noticeably faster than the Python version even though we did not try to optimize it at all. This makes sense because Python has a lot of overhead per operation and assembly does not. For N=100 the Python version was visibly slower in updating frames.

# IX. CONCLUSION

Milestone 2 was significantly harder than milestone 1. Writing the simulation in assembly requires understanding every single step that the processor takes, there is no abstraction hiding anything from you. We spent a lot of time on register management and debugging issues that would never exist in Python.

The thing we learned the most is how much work happens automatically in a high level language. Things like keeping track of variable values, computing array indices, saving state across function calls, all of this is done for you in Python. In assembly you do all of it manually and one mistake anywhere can break the whole thing.

The SIMD exercises were interesting because they showed a completely different way of thinking about loops. Instead of iterating over elements one by one you think about doing operations on whole arrays at once. This is clearly much more efficient for the kind of calculations in our simulation.

In the future it would be interesting to vectorize the inner loop of calc_acc using RVV instructions. Based on the profiling results this is where almost all the computation happens and a 4x to 8x speedup there would make a big difference for larger simulations.

# REFERENCES

[1] "RISC-V Instruction Set Manual," RISC-V International. [Online]. Available: https://riscv.org/specifications/

[2] "RISC-V Vector Extension Specification." [Online]. Available: https://rvv-isadoc.readthedocs.io/en/latest/index.html

[3] "mmap - Memory-mapped file objects," Python 3 Documentation. [Online]. Available: https://docs.python.org/3/library/mmap.html

[4] "QEMU Plugin Interface," QEMU Documentation. [Online]. Available: https://www.qemu.org/docs/master/devel/plugins.html

[5] "SIMD Instructions Considered Harmful," SIGARCH. [Online]. Available: https://www.sigarch.org/simd-instructions-considered-harmful/

[6] "Leapfrog Integration," Wikipedia. [Online]. Available: https://en.wikipedia.org/wiki/Leapfrog_integration